# Westbrook City Design Criteria Excerpt

Manual excerpt for Westbrook Avenue reconstruction project. Source: City Department of Public Works design criteria binder, revision 2026-04.

# 1. Drainage design criteria

| source_id | topic | criterion | value | unit | application note |
| --- | --- | --- | --- | --- | --- |
| CRIT-D1 | design storm | urban collector sag point storm recurrence | 25 | year | Applies where flooding at a sag point can pond against the curb or travel lane. |
| CRIT-D2 | design storm | on-grade inlet storm recurrence | 10 | year | Applies only where bypass flow has a continuous downstream overland path. |
| CRIT-D3 | allowable spread | maximum spread at urban collector sag | 2.4 | m | Spread measured from curb face into the travel lane. |
| CRIT-D4 | ponding depth | maximum allowable sag ponding depth | 0.15 | m | Measured above low gutter elevation. |
| CRIT-D5 | pipe slope | minimum slope for 375 mm RCP | 0.50 | percent | Applies to new storm pipe. |
| CRIT-D6 | pipe slope | minimum slope for 450 mm RCP | 0.40 | percent | Applies to new storm pipe. |
| CRIT-D7 | pipe slope | minimum slope for 600 mm RCP | 0.25 | percent | Applies to downstream trunk pipes. |
| CRIT-D8 | hydraulics | Manning roughness for RCP | 0.013 | unitless | Use for design capacity unless inspection shows severe roughness. |

# 2. IDF rainfall intensities and rational method parameters

| source_id | return period | Tc 10 min intensity mm/h | Tc 12 min intensity mm/h | Tc 15 min intensity mm/h | note |
| --- | --- | --- | --- | --- | --- |
| CRIT-I1 | 10-year | 78 | 72 | 66 | Preliminary on-grade inlet check only. |
| CRIT-I2 | 25-year | 99 | 91 | 84 | Required for Westbrook Avenue sag point checks. |
| CRIT-I3 | 50-year | 115 | 106 | 98 | Use only for overflow sensitivity review. |

CRIT-I4: For mixed commercial/residential frontage with connected impervious area, use a minimum runoff coefficient C = 0.78 unless a lower value is supported by land-use mapping.

CRIT-I5: The rational method conversion is Q = 0.00278 C i A where Q is m3/s, i is mm/h, and A is hectares.

CRIT-I6: Hydrology workbook formulas control only when rainfall intensity, runoff coefficient, catchment area, and return period are consistent with this criteria excerpt.

# 3. ADA ramp criteria

| source_id | feature | maximum running slope pct | maximum cross slope pct | maximum landing slope pct | application note |
| --- | --- | --- | --- | --- | --- |
| CRIT-A1 | curb ramp run | 8.33 | 2.00 | not applicable | Running slope above 8.33 percent is noncompliant. |
| CRIT-A2 | ramp landing | 2.00 | 2.00 | 2.00 | Landing slope is checked both longitudinally and transversely. |
| CRIT-A3 | detectable warning panel | 2.00 | 2.00 | 2.00 | Panel must match landing cross-slope criteria. |

# 4. Pavement rehabilitation and subgrade triggers

| source_id | criterion | threshold | required classification |
| --- | --- | --- | --- |
| CRIT-P1 | Soaked subgrade CBR below 4 percent | CBR < 4 | Full-depth reconstruction or undercut/stabilization. |
| CRIT-P2 | FWD D0 deflection above 0.65 mm | D0 > 0.65 mm | Mill-and-overlay is not acceptable without structural repair. |
| CRIT-P3 | Pavement PCI above 60 with D0 below 0.55 mm | PCI > 60 and D0 < 0.55 | Mill-and-overlay may be acceptable if no geotechnical trigger exists. |
| CRIT-P4 | Station-specific geotechnical evidence conflicts with network summary | station-specific evidence controls | Use boring and laboratory CBR closest to the station range. |

# 5. Utility clearance criteria

| source_id | criterion | minimum value | condition for acceptance |
| --- | --- | --- | --- |
| CRIT-U1 | vertical clearance between storm sewer and gas main | 0.30 m | Clearance must be based on confirmed gas depth at the conflict station. |
| CRIT-U2 | vertical clearance between storm sewer and water service | 0.30 m | Owner may approve smaller clearance only by written record. |
| CRIT-U3 | unverified utility depth | not acceptable | Assumed cover or paint-only markout cannot close a vertical clearance conflict. |
| CRIT-U4 | utility markout age | 30 days | Markout must be renewed if construction occurs after expiration. |

# 6. Source precedence note

CRIT-S1: This criteria excerpt controls design thresholds and acceptance limits. It does not control measured field elevations.

CRIT-S2: Signed survey and inspection records control measured elevations, observed condition, and field invert data.

CRIT-S3: Public complaints and photos support location and timing of observed flooding, but they do not determine recurrence interval or design-storm requirements.

CRIT-S4: Utility clearance is unresolved when depth is missing, even if the preliminary profile includes an assumed cover value.