# Westbrook Avenue Geotechnical Boring Summary

Project: Westbrook Avenue reconstruction and drainage upgrade. Report date: 2026-05-24. Prepared for municipal public works project file.

Coordinate and station basis: SC-2026-REV-B station control. Boring locations are approximate within the accuracy noted in the field logs.

# 1. Investigation scope and use limits

Six shallow pavement/subgrade borings were advanced between Station 0+480 and Station 0+840 to characterize the pavement support conditions near Pine Street and the Westbrook Avenue sag point.

The boring information is suitable for station-range rehabilitation classification, subgrade risk screening, and comparison with pavement cores and FWD results. It is not a utility-depth record and does not verify gas-main clearance.

Where station-specific borings conflict with network-level pavement summaries, the station-specific boring and laboratory CBR evidence should control the subgrade classification for the affected station range.

# 2. Boring location and subsurface summary

| source_id | boring_id | station | offset | pavement thickness mm | base thickness mm | subgrade material | groundwater note | field classification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO-B1 | B-04 | 0+520.0 | -1.0 | 110 | 165 | sandy silt with gravel | no seepage to 2.5 m | medium support |
| GEO-B2 | B-05 | 0+580.0 | 0.8 | 105 | 150 | silty sand | moist at 1.9 m | medium to good support |
| GEO-B3 | B-06 | 0+640.0 | -0.6 | 88 | 118 | organic silt with trace clay | seepage at 1.4 m | weak support |
| GEO-B4 | B-07 | 0+720.0 | 0.5 | 92 | 110 | soft silty clay with organics | water at 1.2 m | weak support |
| GEO-B5 | B-08 | 0+800.0 | -0.4 | 98 | 150 | silty sand with gravel | no seepage to 2.2 m | medium support |
| GEO-B6 | B-09 | 0+840.0 | 0.7 | 112 | 170 | sand and gravel | no seepage to 2.5 m | good support |

# 3. Laboratory CBR and moisture results

| source_id | boring_id | sample depth m | natural moisture pct | soaked CBR pct | swell pct | interpretation |
| --- | --- | --- | --- | --- | --- | --- |
| GEO-L1 | B-04 | 0.45-0.90 | 11.8 | 7.6 | 0.4 | Suitable for mill-and-overlay if pavement FWD response is acceptable. |
| GEO-L2 | B-05 | 0.45-0.90 | 10.9 | 8.1 | 0.3 | This value is not representative of the 0+610 to 0+760 soft zone. |
| GEO-L3 | B-06 | 0.45-0.90 | 22.4 | 3.2 | 1.8 | CBR below 4; full-depth reconstruction or undercut stabilization criterion is triggered. |
| GEO-L4 | B-07 | 0.45-0.90 | 24.1 | 2.8 | 2.3 | CBR below 4; confirms soft subgrade zone between 0+610 and 0+760. |
| GEO-L5 | B-08 | 0.45-0.90 | 14.2 | 5.9 | 0.6 | Support improves east of station 0+780. |
| GEO-L6 | B-09 | 0.45-0.90 | 9.8 | 9.4 | 0.2 | No soft subgrade trigger. |

# 4. Subgrade zone interpretation

GEO-N1: The station range 0+610 to 0+760 contains two weak borings with soaked CBR below 4 percent and elevated moisture. That range should not be classified from B-05 alone.

GEO-N2: The pavement summary row that extends the B-05 CBR of 8.1 into the 0+610 to 0+760 station range is not geotechnically defensible.

GEO-N3: Water-bearing silt and soft silty clay were encountered in the same station range where FWD deflection records exceed the surface-only rehabilitation threshold.

GEO-N4: No boring was advanced exactly at Station 0+735.4. The subgrade classification for the sag point must be interpolated from B-06 and B-07 and should remain flagged for field verification during excavation.

# 5. Construction risk notes

| source_id | risk item | station range | evidence | record status |
| --- | --- | --- | --- | --- |
| GEO-R1 | soft subgrade under proposed drainage trench | 0+610 to 0+760 | B-06 and B-07 CBR below 4 with moisture above 22 percent | resolved as weak support zone |
| GEO-R2 | groundwater entering trench excavation | 0+640 to 0+720 | seepage at 1.4 m and water at 1.2 m | unresolved until excavation observations are logged |
| GEO-R3 | mismatch with pavement network summary | 0+600 to 0+800 | network summary uses B-05 CBR 8.0 for broader range | conflict requires pavement/geotech reconciliation |
| GEO-R4 | gas-main clearance | 0+820 | borings do not provide utility depth | not controlled by geotechnical report |